ACCESSIBLE EDUCATIONAL TERMINAL

WITH TACTILE MORSE INPUT & INTEGRATED SCANNER

────────────────────────────────────────────────

Complete Project Master Plan  |  Zero to Hero Guide

| Project Code | CSR-DES-INFINEON-2025 |
| --- | --- |
| Prepared by | Dhruv Vaghela & Dax Patel |
| Mentor | Rajiv Sir |
| Reference Repo | github.com/parth-2005/morse-ai |
| Version | 1.0 (Final Master Plan) |
| Date | 13 May 2026 |

# SECTION 1: PROBLEM STATEMENT

Visually impaired individuals face enormous barriers when interacting with modern technology and their surrounding information environment. More than 285 million people worldwide are visually impaired, of whom 39 million are completely blind. Despite rapid technological progress, the tools available to blind users remain fragmented, heavy, and expensive.

### 1.1  Core Problems Identified

- TEXT INACCESSIBILITY: Blind users cannot read printed text on books, posters, product labels, notices boards, or digital screens without external human assistance.

- NO NATURAL INPUT METHOD: Standard keyboards are designed for sighted users. Blind people cannot type efficiently or give input to computers or AI systems independently.

- LANGUAGE BARRIER: Most assistive tools only support English. Users who speak Gujarati or Hindi have no native-language solution.

- FRAGMENTED & HEAVY HARDWARE: Existing solutions (Braille displays, screen readers, separate cameras) require multiple disconnected peripherals — causing user fatigue, misalignment errors, and carrying difficulty.

- NO AI INTEGRATION: Current blind assistive devices do not allow the user to interact with modern AI (like ChatGPT) for education, queries, or assistance.

- HIGH COST: Commercial assistive devices are priced beyond reach for most students in India.

ℹ️ Project Trigger This project was originally started by Parth Panot and team, who designed the hardware enclosure and filed a design patent. Rajiv Sir has now handed the project to Dhruv Vaghela and Dax Patel's team to complete all software implementation, integration, testing, and final delivery.

# SECTION 2: OUR FULL SOLUTION

We are building a self-contained, affordable Accessible Educational Terminal powered by Raspberry Pi. It is enclosed in a custom 3D-printed two-part shell with honeycomb ventilation. The device unifies all essential functions blind users need — reading, typing, and AI interaction — into a single compact unit.

### 2.1  Core Functionalities at a Glance

| Feature | What it Does |
| --- | --- |
| OCR Text Reader | Camera captures text from physical documents, books, posters. Tesseract OCR extracts it. pyttsx3 / gTTS reads it aloud in Gujarati, Hindi, or English. |
| Morse Code Input | 3 tactile raised buttons on top of device. User taps dots and dashes to spell letters. System decodes and types text — no keyboard needed. |
| Voice-to-Text Input | User speaks; Google Speech Recognition converts voice to text. Secondary input method alongside Morse. |
| ChatGPT Integration | Typed or spoken queries are sent to OpenAI API. AI responds. Response is read aloud — giving the blind user a full AI assistant. |
| Multilingual TTS | Text-To-Speech output in English, Gujarati, and Hindi. Language auto-detected or user-selectable. |
| Translation Mode | Captured or typed text can be translated to another language before reading aloud. |
| Haptic/Audio Feedback | Every button press gives audio beep confirmation. Wrong input gives different tone. |
| Standalone Device | No laptop needed. Runs entirely on Raspberry Pi 4. Portable, battery-optional. |
| Hand Gesture Control | USB webcam detects 21 hand landmarks via MediaPipe. Open palm = Scan, Thumbs up = Confirm, Two fingers = Voice mode, Fist = Stop. Eliminates physical button dependency. |
| Emotion-Adaptive AI | librosa analyses pitch, tempo and MFCCs from user voice. If user sounds confused or stressed, device auto-slows TTS and simplifies AI responses. |
| Offline AI (Local LLM) | TinyLlama 1.1B model runs entirely on Pi via llama.cpp. No internet required. Full AI assistant works in villages and schools with zero connectivity. |
| Bone Conduction Audio | Bone conduction transducer vibrates sound through skull — ears stay open to hear environment. Safer outdoors. Works for users with partial hearing loss. |
| GPS Navigation Assistant | NEO-6M GPS module provides live location. User asks 'Where am I?' or 'How do I reach the hospital?' — AI gives turn-by-turn directions read aloud. |
| Object Recognition (YOLO) | YOLOv8-nano model identifies real-world objects via camera — medicine bottles, currency notes, persons, obstacles — all described aloud within 1 second. |
| 🔒 Confidential Mode | Before every scan, device asks: 'Confidential or Normal?' — If confidential (bank OTP, account number, passwords), audio plays ONLY through bone conduction transducer (inaudible to bystanders). Normal content plays through speaker. |

### 2.2  New Improvements Added by Our Team

- Full ChatGPT / AI assistant integration (not in original plan)

- Multilingual support — Gujarati + Hindi + English TTS and translation

- Voice-to-text as secondary input alongside Morse code

- Real-time audio feedback (beeps, confirmation tones) for every user action

- Modular software architecture — each function (OCR, Morse, Voice, AI) is an independent Python module

- Safety shutdown command via Morse code sequence

- Hand Gesture Recognition — user controls all device modes without touching any button

- Emotion Detection via Voice — device adapts its response speed and complexity based on user's emotional state

- Fully Offline AI via TinyLlama 1.1B — no internet, no API costs, works anywhere

- Bone Conduction Audio Output — ears stay free, safer for outdoor navigation

- GPS-based Navigation Assistant — real-time location + AI-generated walking directions

- Object Recognition via YOLOv8-nano — device becomes a visual interpreter of the world

- Confidential Mode — dual audio routing system (bone conduction for private data, speaker for normal). Device asks user before every scan whether content is sensitive.

# SECTION 3: COMPLETE HARDWARE LIST

IMPORTANT: All items below must be purchased in one batch. Do not miss any item. Even small components like resistors are critical for hardware protection.

🚨 RED ALERT: One-Time Purchase Rajiv Sir allows only ONE purchasing opportunity. Verify every row below before submitting the order. Adding even small missing parts later will delay the entire project.

| ✅ GREEN ROW = ALREADY OWNED (do not purchase) | 🛒 ORANGE ROW = NEED TO BUY (add to purchase list) |
| --- | --- |

### 3.1  Main Processing & Core Hardware

| Component | Specification / Model | Purpose & Description |
| --- | --- | --- |
| ✅ Raspberry Pi 5 (8GB RAM) | Official Pi 5 — 8GB RAM, 2.4GHz quad-core | ALREADY OWNED. Main brain of device. Pi 5 is 2-3x faster than Pi 4 — better for TinyLlama offline AI and YOLOv8 object detection. |
| 🛒 Camera Module 3 (12MP, Autofocus) | Sony IMX708, HDR, Phase-detect autofocus — buy with 22-pin cable | NEED TO BUY. For OCR and Object Recognition. Pi 5 uses 22-pin FPC connector — Camera Module 3 standard box now includes both 15-pin AND 22-pin cable. Buy from robu.in or amazon.in (~₹3,500). |
| ✅ MicroSD Card (64GB) | Raspberry Pi branded 64GB MicroSD — already inside Pi 5 | ALREADY OWNED. Stores Pi OS, all Python code, and system files. 64GB is sufficient. |
| ✅ Official 27W USB-C Power Supply | Raspberry Pi 27W USB-C AC Adaptor (5V/5A) | ALREADY OWNED. Correct supply for Pi 5 — Pi 5 needs 5A, Pi 4 only needed 3A. Do not use any other charger. |
| ✅ Official Active Cooler for Pi 5 | Raspberry Pi Active Cooler — fan + heatsink combined, 4-pin JST | ALREADY OWNED and already mounted. Connects to Pi 5 fan header (4-pin JST connector on board) — NOT to GPIO. No separate heatsinks needed. |

### 3.2  Input Hardware — Morse Code & Pico W

ℹ️ Architecture Note: Pico W as Morse Handler Your Raspberry Pi Pico W handles all 3 Morse buttons directly via MicroPython. It decodes dots, dashes, and spaces in real time and sends decoded letters to Pi 5 via USB serial. This offloads real-time button timing from Pi 5 — making Morse decoding faster and more reliable.

| Component | Specification / Model | Purpose & Description |
| --- | --- | --- |
| ✅ Raspberry Pi Pico W | RP2040 microcontroller with WiFi — already on breadboard | ALREADY OWNED. Dedicated Morse code button handler. Runs MicroPython. Sends decoded letters to Pi 5 via USB serial cable. Breadboard already included. |
| 🛒 12mm Dome Tactile Buttons | 12mm raised dome, through-hole, 4-pin, qty: 10 | NEED TO BUY. Your current 6x6mm buttons are too small for blind users to locate reliably. 12mm raised dome is essential. Buy 10 — use 3, keep 7 as spares. flyrobo.in (~₹15 each). |
| 🛒 10kΩ Resistors | Through-hole, 1/4W, qty: 20 | NEED TO BUY. Pull-down resistors for Pico W GPIO button inputs — prevents floating signals. flyrobo.in (~₹30 for 20pcs). |
| 🛒 Jumper Wire Combo Kit | 120pcs M-M + M-F + F-F, 20cm Dupont wires | NEED TO BUY. Connecting buttons to Pico W, Pico W to Pi 5, GPS to Pi 5, PAM8403 to audio card. flyrobo.in (~₹150-200). |
| ✅ 830-point Breadboard (large) | Full-size 830-tie breadboard — already have | ALREADY OWNED. Large enough for Pico W + buttons + resistors + GPS module testing layout. |
| ✅ Active + Passive Buzzers | HXD Active buzzer + Passive buzzer, 2 pcs | ALREADY OWNED. Used for audio beep feedback on each button press. Active buzzer = constant beep, Passive = tone control via PWM. |
| ✅ LM358P Op-Amp IC | Dual op-amp DIP-8, Texas Instruments | ALREADY OWNED. Use as audio signal buffer between USB audio card output and PAM8403 amplifier input for bone conduction. |

### 3.3  Audio Output Hardware

| Component | Specification / Model | Purpose & Description |
| --- | --- | --- |
| 🛒 USB Audio Card (sound dongle) | Generic USB-A sound card dongle, stereo 3.5mm out + mic in | NEED TO BUY. Pi 5 has NO 3.5mm audio jack — audio only available via HDMI or USB. Your USB-C to 3.5mm adapter is phone-type and won't work here. Buy a USB-A audio dongle. amazon.in (~₹150-250). |
| 🛒 Mini Speaker (3W amplified) | 3W USB-powered or 3.5mm amplified compact speaker | NEED TO BUY. Plays TTS speech output. Must be a powered/amplified speaker — Pi 5 cannot drive passive speakers directly. amazon.in (~₹250-500). |
| 🛒 Earphones / Headset | Standard 3.5mm earphones | NEED TO BUY. For private TTS listening by blind user. Connects to USB audio card's headphone jack. (~₹100-200). |
| 🛒 3.5mm Audio Cable (M-M) | 30cm stereo 3.5mm cable | NEED TO BUY. Connects USB audio card output to PAM8403 amplifier input for bone conduction circuit. (~₹50-80). |

### 3.4  Enclosure & Assembly Hardware

| Component | Specification / Model | Purpose & Description |
| --- | --- | --- |
| 🛒 3D Printed Shell — Top Part | Orange PLA/ABS — use patent drawings for dimensions | NEED TO PRINT. Top shell with 3 button holes (12mm), camera aperture, honeycomb vents. Pi 5 PCB footprint is same as Pi 4 — patent design dimensions still valid. |
| 🛒 3D Printed Shell — Base Plate | Dark Navy PLA/ABS | NEED TO PRINT. Bottom enclosure. Pi 5 mounting holes are same position as Pi 4 — standoffs and screws unchanged. |
| 🛒 M2.5 x 6mm Screws + Standoffs | Stainless M2.5 screws qty:10 + M2.5 brass standoffs 10mm qty:4 | NEED TO BUY. Mounts Pi 5 inside the base plate. Pi 5 board has same 4 mounting holes as Pi 4. (~₹100-200). |
| 🛒 M3 x 10mm Screws | Stainless, qty: 8 | NEED TO BUY. Fastens top shell to base plate. (~₹50-100). |
| 🛒 Foam / Rubber Padding | 5mm adhesive foam sheets | NEED TO BUY. Vibration dampening inside shell. (~₹40-80). |

### 3.5  Connectivity & Setup Hardware

| Component | Specification / Model | Purpose & Description |
| --- | --- | --- |
| 🛒 USB Keyboard & Mouse | Standard USB set (for initial setup only) | NEED TO BUY if you don't already own one. Used ONLY for Pi 5 initial OS setup — not used during normal device operation. |
| 🛒 Micro-HDMI to HDMI Cable | Micro-HDMI (Type D) to HDMI-A — Pi 5 has 2x micro-HDMI ports | NEED TO BUY. Pi 5 uses Micro-HDMI, NOT full-size HDMI. Required to connect Pi 5 to any monitor during setup. amazon.in (~₹200-350). |
| 🛒 USB Microphone (Pi compatible) | USB plug-and-play condenser mic for Raspberry Pi | NEED TO BUY. For voice-to-text input. Plugs into Pi 5 USB-A port. flyrobo.in has one specifically for Pi (~₹250-400). |
| 🛒 Powered USB Hub (4-port) | 5V powered USB hub with own power adapter | NEED TO BUY. Pi 5 USB ports have limited total current. Hub needed to simultaneously run: USB mic + USB audio dongle + USB webcam + USB flash drive. amazon.in (~₹400-800). |
| 🛒 Ethernet Cable (Cat5e, 1m) | Standard RJ45 Cat5e straight cable | NEED TO BUY. Wired internet for initial package downloads — more reliable than WiFi for 2GB+ TinyLlama model download. (~₹80-150). |
| 🛒 Anti-static Wrist Strap | ESD wrist strap with crocodile clip | NEED TO BUY. Pi 5 BCM2712 chip is sensitive to static. Essential during assembly. amazon.in (~₹60-150). |
| 🛒 Multimeter (basic) | Digital multimeter, DC/AC | NEED TO BUY if not owned. Test GPIO voltages, GPS 3.3V supply, PAM8403 5V supply, button continuity. (~₹300-500). |

### 3.6  Gesture Recognition Hardware

| Component | Specification / Model | Purpose & Description |
| --- | --- | --- |
| 🛒 USB Webcam (720p HD) | Logitech C270 or equivalent 720p USB webcam | NEED TO BUY. Dedicated camera for MediaPipe hand gesture detection. Keeps Pi Camera 3 free for OCR simultaneously. amazon.in (~₹600-1,200). |
| 🛒 Webcam Mount / Clip | Flexible gooseneck or clip-on webcam holder | NEED TO BUY. Positions webcam consistently at user's hand zone. Critical for reliable gesture angle. (~₹100-200). |
| 🛒 USB Extension Cable (A-A, 50cm) | USB 2.0 A-male to A-female extension cable | NEED TO BUY. Positions webcam away from device body at correct gesture capture angle. (~₹50-80). |

### 3.7  GPS Navigation Hardware

ℹ️ GPS Module Note NEO-8M is recommended over NEO-6M. The NEO-8M at flyrobo.in (₹993) has a built-in ceramic antenna — no separate external antenna or SMA adapter needed. It also supports more satellite constellations (GPS + GLONASS + BeiDou) for faster fixes in India.

| Component | Specification / Model | Purpose & Description |
| --- | --- | --- |
| 🛒 NEO-8M GPS Module | NEO-8M STM32 with built-in ceramic antenna, UART output | NEED TO BUY. Provides real-time GPS coordinates. Built-in ceramic antenna — no external antenna needed. Connects to Pi 5 UART (GPIO 14/15). flyrobo.in ₹993. Better than NEO-6M. |
| 🛒 Jumper Wires (M-F, short 15cm) | Extra 10pcs M-F 15cm Dupont wires | NEED TO BUY. UART TX, RX, 3.3V, GND — 4 wires from NEO-8M to Pi 5 GPIO. Can reuse from jumper combo kit. |

### 3.8  Bone Conduction Audio Hardware

| Component | Specification / Model | Purpose & Description |
| --- | --- | --- |
| 🛒 Bone Conduction Transducer (8Ω) | 8Ω 1W bone conduction driver/transducer module | NEED TO BUY. Vibrates sound through skull — ears stay open to hear environment. Safer outdoors than earphones. Connect output of PAM8403 amp to transducer. amazon.in (~₹300-700). |
| 🛒 PAM8403 Amplifier Board (with pot) | PAM8403 Mini 5V digital amp board with switch potentiometer | NEED TO BUY — qty: 2 (one spare). Amplifies audio signal from USB audio card to drive bone conduction transducer. 5V from Pi 5 GPIO powers it. flyrobo.in (~₹60-80 each). |
| ✅ LM358P Op-Amp IC | Already owned — used as audio signal buffer | ALREADY OWNED. Used as signal buffer between USB audio card 3.5mm output and PAM8403 input to protect audio quality. |
| 🛒 Elastic Temple Clip / Head Strap | Adjustable silicone strap or headphone frame clip | NEED TO BUY. Holds bone conduction transducer against user's temple/cheekbone. (~₹100-200). |

### 3.9  Object Recognition & Offline AI Hardware

| Component | Specification / Model | Purpose & Description |
| --- | --- | --- |
| 🛒 Google Coral USB Accelerator | USB 3.0 Edge TPU — plugs into Pi 5 USB 3.0 (blue) port | NEED TO BUY (optional but strongly recommended). Runs YOLOv8 object detection 5-10x faster than Pi 5 CPU alone. Pi 5 is compatible. amazon.in (~₹3,500-5,000). |
| 🛒 USB 3.0 Flash Drive (128GB) | SanDisk Ultra USB 3.0 128GB or equivalent | NEED TO BUY. Stores TinyLlama 1.1B model (~700MB) and YOLOv8 weights. Keep models off MicroSD to avoid wear. Plug into Pi 5 USB 3.0 port. amazon.in (~₹700-1,100). |
| ✅ 64GB MicroSD (already in Pi 5) | Raspberry Pi branded 64GB — sufficient | ALREADY OWNED. No upgrade needed — 64GB with models on USB flash drive is more than enough. |

### 3.10  Critical Spare Parts — Do NOT Skip

| Component | Specification / Model | Purpose & Description |
| --- | --- | --- |
| 🛒 Spare Camera Cable (22-pin FPC) | Raspberry Pi 5 camera cable — 22-pin to 15-pin FPC, 200mm | NEED TO BUY. Pi 5 uses 22-pin FPC connector (different from Pi 4). Camera ribbon tears during assembly. One spare saves the whole project. amazon.in/robu.in (~₹80-150). |
| 🛒 Spare MicroSD Card (32GB) | Class 10 / A1 or better, any brand | NEED TO BUY. Backup OS image. If primary 64GB SD corrupts, restore from backup in 20 minutes. (~₹300-450). |
| 🛒 Spare 12mm Buttons (5pcs) | Same 12mm dome tactile button as in 3.2 | NEED TO BUY alongside main batch. Heavy tactile use by blind users wears buttons out. Spares prevent project-stopping failures. |
| 🛒 Heat Shrink Tube Set | Assorted 2mm-6mm, 1 pack | NEED TO BUY. Insulate all exposed wire joints inside enclosure. Prevents short circuits. flyrobo.in/amazon.in (~₹60-120). |
| 🛒 Thermal Paste or Thermal Pads | Small syringe thermal paste or 4-pack pads | NEED TO BUY if not using official Pi 5 active cooler thermal pads — the official cooler already includes pre-applied thermal pad. Only needed for additional components. (~₹50-100). |

### 3.6  Gesture Recognition Hardware

| Component | Specification / Model | Purpose & Description |
| --- | --- | --- |
| USB Webcam (HD) | Logitech C270 or equivalent 720p USB webcam | Dedicated camera for hand gesture detection via MediaPipe. Keeps the Pi Camera free for OCR simultaneously. |
| Webcam Mount / Clip | Flexible gooseneck USB webcam mount | Positions webcam to face user's hand area consistently. Critical for reliable gesture detection angle. |
| USB Extension Cable (A-A) | 50cm USB 2.0 extension cable | Positions webcam away from body of device at correct angle for hand gesture capture. |

### 3.7  GPS Navigation Hardware

| Component | Specification / Model | Purpose & Description |
| --- | --- | --- |
| GPS Module | NEO-6M or NEO-M8N UART GPS module | Provides real-time latitude/longitude coordinates. Connects to Raspberry Pi via UART serial (TX/RX pins). NEO-M8N is faster to get satellite fix. |
| GPS Ceramic Antenna | Active patch antenna with SMA connector, 3-5V | Improves GPS satellite signal reception significantly. The small built-in antenna on NEO-6M alone is weak indoors. |
| SMA to uFL Adapter | SMA female to uFL/IPEX adapter cable | Connects the external GPS antenna to the NEO module's uFL connector. |
| Jumper Wires (M-F) | 15cm, qty: 4 extra | UART TX, RX, 3.3V, GND connections from GPS module to Pi GPIO pins. |

### 3.8  Bone Conduction Audio Hardware

| Component | Specification / Model | Purpose & Description |
| --- | --- | --- |
| Bone Conduction Transducer | 8Ω 1W bone conduction driver module | Vibrates against skull bone — transmits sound without blocking ears. User hears TTS while remaining aware of surroundings. Connect via 3.5mm audio output. |
| Small PAM8403 Amplifier Board | PAM8403 5V 3W stereo class-D amp module | Bone conduction transducers need amplification. PAM8403 module (₹40) powers the transducer from Pi's 5V GPIO pin. |
| Elastic Head Strap / Temple Clip | Adjustable silicone strap or temple clip | Holds bone conduction transducer against user's temple or cheekbone during use. Can be adapted from a spare pair of headphone frames. |
| 3.5mm Stereo Cable (short) | 15cm 3.5mm male to bare wire ends | Connects USB audio card output to the PAM8403 amplifier board input. |

### 3.9  Object Recognition & Offline AI Hardware

| Component | Specification / Model | Purpose & Description |
| --- | --- | --- |
| Google Coral USB Accelerator | USB 3.0 Edge TPU accelerator | Runs YOLOv8 object detection at 5-10x speed vs CPU alone. Plugs into USB 3.0 port on Pi. Makes real-time object detection practical on Pi 4. |
| USB 3.0 Flash Drive (128GB) | SanDisk Ultra USB 3.0 128GB | Stores TinyLlama 1.1B model file (~2GB) and YOLOv8 weights (~6MB). Keep model files off MicroSD to avoid wear. Faster read speed than SD card. |
| MicroSD Card (64GB) | Samsung Pro Endurance 64GB | Upgrade from 32GB — extra space for offline model caching, logs, captured images during OCR sessions. |

### 3.10  Critical Spare Parts — Do NOT Skip

| Component | Specification / Model | Purpose & Description |
| --- | --- | --- |
| Camera Ribbon Cable (spare) | Pi Camera v2 compatible 15cm ribbon | Camera ribbon tears easily during assembly. Having 1 spare prevents a project-stopping failure. |
| Spare MicroSD Card | 16GB minimum, Class 10 | Backup OS image. If primary SD corrupts, load backup and resume within 30 minutes. |
| Spare Tactile Buttons | 12mm dome, qty: 5 extra | Buttons fail with heavy use. Spares allow immediate replacement without re-purchasing. |
| Spare Jumper Wires | Mixed M-F M-M F-F, qty: 30 pcs | Wires break or come loose during assembly. Spares save time and prevent rework. |
| Heat Shrink Tubing Set | Assorted 2mm-6mm diameter, 1 pack | Insulate all exposed wire joints inside the enclosure. Prevents shorts between wires. |
| Thermal Paste / Pads | Small syringe or 4-pack thermal pads | For heatsink-to-chip contact. Better than the sticky tape that comes with cheap heatsink kits. |

# SECTION 4: SOFTWARE, TECHNOLOGY & PLATFORMS

### 4.1  Operating System

| Item | Details |
| --- | --- |
| OS | Raspberry Pi OS (Bookworm, 64-bit) — formerly Raspbian |
| Download Tool | Raspberry Pi Imager (free download from raspberrypi.com) |
| Storage | Written to 32GB MicroSD via laptop using the Imager tool |

### 4.2  Programming Language & Core Libraries

| Library / Tool | Purpose |
| --- | --- |
| Python 3.10+ | Primary programming language for all modules |
| RPi.GPIO | Read button presses from GPIO pins (Morse input) |
| Picamera2 | Control the Raspberry Pi Camera Module v2 and capture images |
| Tesseract OCR (pytesseract) | Convert camera-captured images to text (OCR engine) |
| OpenCV (cv2) | Pre-process images before OCR (binarize, denoise, deskew) to improve accuracy |
| Pillow (PIL) | Image format handling for OCR pipeline |
| pyttsx3 | Offline Text-To-Speech. Works without internet. Gujarati/Hindi support via eSpeak voices. |
| gTTS (Google TTS) | Online TTS with higher quality. Better Gujarati/Hindi pronunciation when internet is available. |
| SpeechRecognition | Voice-to-text using Google Speech API over internet |
| PyAudio | Microphone audio capture for SpeechRecognition library |
| openai (Python SDK) | Send queries to ChatGPT API and receive AI responses |
| googletrans / deep-translator | Translate OCR text or user input between English, Gujarati, Hindi |
| pygame / playsound | Play audio beep feedback tones for button presses |
| threading | Run Morse decoder and TTS simultaneously without blocking each other |
| json / configparser | Store user settings (language preference, API keys, Morse timing thresholds) |
| mediapipe | Google's framework for real-time hand landmark detection (21 points per hand). Powers gesture recognition mode. |
| librosa | Audio analysis — extracts pitch, tempo, and MFCC features from voice recordings for emotion detection. |
| speechbrain | Deep learning speech toolkit — used for emotion classification from extracted librosa audio features. |
| llama-cpp-python | Python bindings for llama.cpp — runs TinyLlama 1.1B language model locally on Raspberry Pi CPU without internet. |
| ultralytics (YOLOv8) | Object detection library — runs YOLOv8-nano model to identify real-world objects via camera in near real-time. |
| pyserial + pynmea2 | pyserial reads raw NMEA data from NEO-6M GPS via UART. pynmea2 parses it into latitude/longitude coordinates. |
| scikit-learn | Provides SVM classifier for emotion detection — trained on librosa features to classify stressed / calm / confused states. |
| numpy | Numerical array operations — required by OpenCV, librosa, YOLOv8, and MediaPipe for data processing. |

### 4.3  Online Services & APIs

- OpenAI API — ChatGPT (GPT-4o or GPT-3.5-turbo) for AI responses. Requires API key.

- Google Speech Recognition API — for voice-to-text. Free tier available.

- Google Translate API (via googletrans library) — for translation features.

- GitHub — for version control of all code (reference: github.com/parth-2005/morse-ai).

- OpenStreetMap / Nominatim API — free, no API key needed. Converts GPS coordinates to street address. Used for 'Where am I?' feature.

- OSRM (Open Source Routing Machine) — free routing API. Generates turn-by-turn navigation directions from GPS coordinates. Works offline with downloaded map tiles.

- HuggingFace Model Hub — download TinyLlama 1.1B GGUF model file for offline AI. One-time download of ~2GB.

### 4.4  Development Tools

- VS Code (on laptop) — code editing with SSH remote extension to Pi

- SSH (Terminal / PuTTY) — remote access to Pi from laptop

- VNC Viewer — remote desktop GUI access to Pi

- Git — version control; push/pull from GitHub repo

# SECTION 5: WHAT IS OUR PROJECT — CORE IDEA & METHODOLOGY

The Accessible Educational Terminal is a standalone, palm-sized computing device built specifically for visually impaired students and users. It is NOT a smartphone. It is NOT a laptop. It is a single-purpose, tactile-first AI-powered device that a blind user can pick up, use without a screen, and interact with entirely through touch, buttons, and audio.

### 5.1  Physical Description

- Rectangular two-part 3D-printed shell (orange top, dark navy base), matching the filed design patent.

- Top surface: 3 raised circular tactile buttons (left side) + 1 large camera aperture (center) + 1 small status LED hole.

- Side panels: honeycomb ventilation matrix for heat management.

- Ports exposed: USB-C power, USB-A (for audio + mic), 3.5mm audio jack area.

- Weight target: under 400g. Portable. Battery-compatible for future upgrade.

### 5.2  How It Works — Core Working Methodology

- User powers on the device via USB-C power supply.

- Raspberry Pi boots Raspberry Pi OS and automatically launches the main Python application.

- Audio welcome message plays: 'Welcome to Accessible Terminal. Ready.'

- User selects MODE via Morse code on the 3 tactile buttons:
      Button 1 = DOT (short press)
      Button 2 = DASH (long press)
      Button 3 = CONFIRM / SPACE / BACKSPACE

- MODE A — SCAN & READ: Device captures image via camera. OCR extracts text. TTS reads it aloud in chosen language.

- BEFORE any audio output: Device first asks 'Is this confidential? Press Button 1 for YES, Button 2 for NO.' — Button 1 → audio routed exclusively to bone conduction transducer (silent to anyone nearby). Button 2 → audio plays through speaker as normal.

- MODE B — TYPE & QUERY: User types using Morse code. Typed text is sent to ChatGPT. AI response read aloud.

- MODE C — VOICE INPUT: User speaks a question. Device transcribes voice, sends to ChatGPT, reads answer back.

- MODE D — TRANSLATE: Captured or typed text is translated before reading.

- MODE E — GESTURE: User waves hand in front of webcam. MediaPipe detects gesture → maps to action (scan / confirm / stop / voice).

- MODE F — OBJECT SCAN: Camera identifies all objects in front of it. Each identified object is named and described aloud: 'I see a water bottle, a book, and a mobile phone.'

- MODE G — NAVIGATE: GPS module reads current coordinates. User asks destination by voice. AI generates step-by-step walking directions which are read aloud one instruction at a time.

- Throughout all modes: Emotion engine monitors voice for stress/confusion. If detected 2 times in a row, device automatically says 'You seem confused. Let me repeat more slowly.' and reduces TTS speed.

### 5.3  The Morse Code Input System — Detailed

Morse code input is the PRIMARY novel feature of this device. It allows a blind user to type any text without a keyboard:

- Button 1 SHORT PRESS = DOT (.)

- Button 2 LONG PRESS (>400ms) = DASH (-)

- Pause >1.5 seconds between presses = letter separator (decode current Morse sequence to a letter)

- Button 3 short press = SPACE between words

- Button 3 long press = BACKSPACE (delete last character)

- Double press Button 3 = CONFIRM / SEND (submit text to AI or read aloud)

The Morse timing thresholds (dot/dash boundary, letter gap, word gap) are configurable in the settings JSON file to match each user's pace.

### 5.4  OCR Pipeline — How Text Reading Works

- User aims device camera at document/poster/book.

- Presses Morse 'scan' sequence or dedicated button shortcut.

- Picamera2 captures a high-resolution image.

- OpenCV pre-processes the image: converts to grayscale, applies adaptive threshold, removes noise, deskews.

- Tesseract OCR (with Hindi/Gujarati language packs) extracts text.

- Detected language is confirmed or user overrides.

- Text is fed into TTS engine and spoken aloud to user.

### 5.5  Hand Gesture Recognition — Detailed

MediaPipe Hands detects 21 landmark points on each hand through the USB webcam in real time. The system classifies gestures by measuring relative distances and angles between landmarks:

- Open Palm (all 5 fingers extended): → MODE A — Scan & Read

- Thumbs Up (thumb extended, others curled): → Confirm / Send

- Two Fingers (index + middle extended): → MODE C — Voice Input

- Fist (all fingers curled): → Stop / Cancel current action

- Point Down (index pointing down): → Repeat last audio output

The gesture system runs on a separate thread alongside the main application — it does not block other operations. Gesture confidence threshold is set to 0.85 to avoid false triggers.

### 5.6  Emotion Detection — Detailed

The emotion engine runs passively every time the user speaks. It does not require a separate trigger:

- librosa.feature.mfcc() extracts 13 MFCC coefficients from each 3-second voice clip.

- librosa.feature.spectral_centroid() and librosa.feature.rms() extract pitch and energy data.

- A pre-trained scikit-learn SVM classifier maps these features to: CALM / STRESSED / CONFUSED.

- If CONFUSED or STRESSED detected in 2 consecutive utterances → TTS speed reduces by 30%, AI prompted with 'Explain in simpler words'.

- If CALM detected → normal speed resumes automatically.

### 5.7  Object Recognition — Detailed

YOLOv8-nano is the smallest version of the YOLO (You Only Look Once) real-time object detection model. On Raspberry Pi 4 with Google Coral USB accelerator, it processes approximately 8-12 frames per second:

- User activates Object mode via Morse code sequence or hand gesture.

- Camera captures a single frame or continuous stream.

- YOLOv8-nano identifies all objects with confidence > 70% and their approximate screen positions (left, center, right, near, far).

- Results converted to natural language: 'In front of you: a chair on the left, a table in the center, a person on the right approximately 2 metres away.'

- Obstacle warning mode: continuous scanning alerts user if a large object appears within close range.

### 5.8  GPS Navigation — Detailed

The NEO-8M GPS module communicates with Raspberry Pi via UART serial at 9600 baud rate:

- pynmea2 library parses NMEA sentences ($GPRMC, $GPGGA) into decimal latitude/longitude.

- Nominatim API (free, no key) reverse-geocodes coordinates to street address.

- When user asks 'Take me to [destination]' by voice, the AI query includes current GPS coordinates and destination.

- AI generates simplified turn-by-turn instructions: 'Walk 200 metres straight, then turn right at the junction.'

- Each instruction is read aloud. After a set time/distance, the next instruction plays automatically.

- GPS fix time: 30-60 seconds outdoors on first boot. Keep the antenna facing open sky.

### 5.9  🔒 Confidential Mode — Detailed

Confidential Mode is a privacy protection system that routes sensitive audio output — bank account numbers, OTPs, passwords, medical records, financial figures — exclusively through the bone conduction transducer. Since bone conduction transmits sound as skull vibration, it is essentially inaudible to anyone standing nearby. The regular speaker stays completely silent during confidential playback.

🔒 HOW CONFIDENTIAL MODE WORKS — STEP BY STEP Step 1: User activates any scan / read / AI response mode as normal (Morse code, voice, or gesture). Step 2: Before any content audio plays, the device ALWAYS asks first — delivered via bone conduction whisper only (not speaker): 'Is this confidential? Press Button 1 for PRIVATE, Button 2 for SPEAKER.' Step 3A — Button 1 pressed (CONFIDENTIAL): Python switches ALSA output to bone conduction device only. Speaker is muted. All content is transmitted silently through skull vibration. Person next to user hears nothing. Step 3B — Button 2 pressed (NORMAL): Python switches ALSA output to USB audio card + speaker. Bone conduction stays silent. Content plays aloud as normal. Step 4: After playback ends, device automatically resets to speaker (normal) mode. The very next scan triggers the same privacy question again — every single time, no exceptions. Step 5: If no button is pressed within 8 seconds, the device defaults to NORMAL (speaker) mode and continues.

The privacy question itself is ALWAYS delivered via bone conduction — so even the question 'Is this confidential?' is never heard by bystanders. This ensures complete end-to-end privacy from prompt to output.

### 5.9.1  Audio Routing — Technical Detail

- Python uses subprocess.run(['amixer', '-D', 'hw:CARD=Device', 'set', 'Master', 'mute']) to silence the speaker before bone conduction playback.

- Two ALSA device IDs are stored in config/settings.json: 'bone_device': 'hw:1,0' and 'speaker_device': 'hw:2,0' — identified once at setup via: aplay -l

- A Python context manager class PrivateAudio() wraps any TTS call: __enter__ mutes speaker + enables bone amp via GPIO pin, __exit__ restores speaker and disables amp.

- Bone conduction amp (PAM8403) enable/disable is controlled via a GPIO output pin — when confidential mode is ON, GPIO pin goes HIGH enabling the amp; when OFF, pin goes LOW cutting bone conduction output.

- This dual-GPIO + ALSA switching ensures there is zero audio bleed between channels — only one output is physically powered at any time.

### 5.9.2  Protected Use Cases

| Sensitive Content Type | Risk If Exposed Through Speaker |
| --- | --- |
| Bank account number on passbook | Overheard by nearby customers in bank branch — fraud risk |
| OTP received via SMS (scanned) | Intercepted by nearby person — SIM/bank account takeover |
| ATM PIN or UPI PIN number | Spoken in public ATM queue — immediate theft risk |
| Medical test report / diagnosis | Private health data heard in clinic waiting room |
| Password written on paper | Read aloud in office — immediate account compromise |
| Salary slip / financial document | Financial details exposed to colleagues or strangers |
| Aadhaar number on card/letter | Identity document number heard by bystanders |

# SECTION 6: STEP-BY-STEP IMPLEMENTATION (ZERO TO HERO)

ℹ️ For Complete Beginners You have zero Raspberry Pi experience — that is perfectly fine. Follow each step exactly in order. Do not skip ahead. Each step builds on the previous one.

## PHASE 1: HARDWARE SETUP & RASPBERRY PI INSTALLATION

STEP 1: Flash Raspberry Pi OS onto MicroSD Card

- On your laptop, go to raspberrypi.com/software and download 'Raspberry Pi Imager'.

- Install and open the Imager application.

- Insert your 32GB MicroSD card into your laptop's card reader (or use a USB card reader).

- In Imager: Choose OS → Raspberry Pi OS (64-bit, Full Desktop).

- Choose Storage → select your MicroSD card.

- Click the gear icon (settings): Set hostname = 'blindterminal', enable SSH, set username='pi' password='blindai123', configure your WiFi SSID and password.

- Click WRITE and wait 10-15 minutes. Imager will flash and verify.

- Safely eject the MicroSD when done.

STEP 2: First Boot & Physical Connection

- Insert the flashed MicroSD card into the slot on the underside of Raspberry Pi 4.

- Connect micro-HDMI cable from Pi to a monitor or TV.

- Connect USB keyboard and USB mouse.

- Connect Ethernet cable from Pi to your WiFi router (for reliable first-time internet).

- Plug in the 5V 3A USB-C power supply LAST. Pi will begin booting.

- After 60-90 seconds, you will see the Raspberry Pi OS desktop.

- Open a terminal (black screen icon on taskbar).

STEP 3: Update the Operating System

- In the terminal, type: sudo apt update

- Press Enter and wait.

- Then type: sudo apt upgrade -y

- This may take 5-15 minutes depending on internet speed. Do not interrupt.

- When done, reboot: sudo reboot

- Wait for Pi to restart.

🚨 RED ALERT: Power During Update Never unplug power during system update or upgrade. Corruption of the OS can make Pi unbootable. If it freezes for more than 20 minutes, wait — do not unplug.

STEP 4: Enable Camera Interface

- Open terminal and type: sudo raspi-config

- Navigate with arrow keys to: Interface Options → Camera → Enable.

- Select OK and then Finish. When asked to reboot, select Yes.

- After reboot, verify camera works: libcamera-hello

- A 5-second camera preview window should appear. If you see it, camera is working.

STEP 5: Enable SSH & Remote Access (Optional but Recommended)

- SSH allows you to control the Pi from your laptop without needing a monitor.

- In raspi-config, go to: Interface Options → SSH → Enable.

- Find your Pi's IP address: hostname -I

- On your laptop, open terminal and type: ssh pi@[Pi's IP address]

- Enter password: blindai123. You now control the Pi remotely.

## PHASE 2: INSTALL ALL SOFTWARE DEPENDENCIES

STEP 6: Install System Packages

- sudo apt install -y python3-pip python3-dev

- sudo apt install -y tesseract-ocr tesseract-ocr-hin tesseract-ocr-guj tesseract-ocr-eng

- sudo apt install -y libopencv-dev python3-opencv

- sudo apt install -y espeak espeak-ng libespeak-ng-dev

- sudo apt install -y portaudio19-dev python3-pyaudio

- sudo apt install -y python3-pygame alsa-utils

- These commands install OCR engine with Gujarati+Hindi language packs, audio tools, and image processing.

STEP 7: Install Python Libraries

- pip3 install pytesseract Pillow opencv-python

- pip3 install pyttsx3 gTTS playsound

- pip3 install SpeechRecognition pyaudio

- pip3 install openai

- pip3 install googletrans==4.0.0-rc1

- pip3 install RPi.GPIO

- pip3 install picamera2

- Wait for all installations to complete. Total ~5-10 minutes.

STEP 8: Verify All Installations

- Type: python3 -c "import cv2; print('OpenCV OK')"

- Type: python3 -c "import pytesseract; print('OCR OK')"

- Type: python3 -c "import openai; print('OpenAI OK')"

- Type: python3 -c "import RPi.GPIO as GPIO; print('GPIO OK')"

- If any module shows an ImportError, re-run its pip3 install command.

## PHASE 3: HARDWARE WIRING — BUTTONS & AUDIO

🚨 RED ALERT: Electrostatic Discharge Risk Always wear your anti-static wrist strap before touching the Raspberry Pi GPIO pins or any component. Touch the metal part of the Pi's USB port frame to discharge yourself first.

STEP 9: Wiring the 3 Tactile Buttons to GPIO

- Identify the GPIO pins on the Pi (use a GPIO diagram — search 'Raspberry Pi 4 GPIO pinout').

- Button 1 (DOT): Connect one leg to GPIO 17 (Pin 11). Connect other leg to GND (Pin 6).

- Button 2 (DASH): Connect one leg to GPIO 27 (Pin 13). Connect other leg to GND (Pin 9).

- Button 3 (CONFIRM/SPACE): Connect one leg to GPIO 22 (Pin 15). Connect other leg to GND (Pin 14).

- Add a 10kΩ pull-down resistor: connect it between each GPIO pin and GND on breadboard.

- Double-check all connections before powering on — reversing polarity won't damage buttons but wrong GPIO wiring can behave unexpectedly.

STEP 10: Connect Audio Hardware

- Plug USB audio card dongle into one of Pi's USB-A ports.

- Connect 3.5mm audio cable from USB audio card's output to the mini speaker input.

- Plug USB microphone into another USB-A port (or powered USB hub).

- In terminal, type: aplay -l   — you should see the USB audio card listed.

- Test speaker: speaker-test -t wav -c 2  (you should hear pink noise from speaker).

- Type Ctrl+C to stop the test.

STEP 11: Connect and Position Camera

- Power OFF the Pi completely before connecting camera.

- Locate the CSI camera ribbon connector on the Pi (between USB and HDMI ports).

- Gently lift the plastic lock on the CSI connector by pulling it upward.

- Slide in the ribbon cable with the BLUE side facing toward USB ports, silver contacts facing the connector.

- Press the plastic lock back down firmly to secure the cable.

- Power the Pi back on and test: libcamera-hello

🚨 RED ALERT: Camera Ribbon Cable The CSI ribbon cable is extremely fragile. Never force it. Never pull on the ribbon — always hold the plastic connector. A torn ribbon cable cannot be repaired and must be replaced.

## PHASE 4: SOFTWARE DEVELOPMENT — CODING THE SYSTEM

ℹ️ Code Repository All reference code is at: github.com/parth-2005/morse-ai — Clone this first: git clone https://github.com/parth-2005/morse-ai.git — Study the existing code before writing new modules.

STEP 12: Set Up Project Folder Structure

- mkdir ~/blindterminal && cd ~/blindterminal

- mkdir modules audio logs config

- Create: config/settings.json — stores language preference, API keys, Morse timing settings.

- Your folder structure: blindterminal/ → main.py, modules/ (morse.py, ocr.py, tts.py, voice.py, ai_query.py, translator.py), audio/ (beep.wav, error.wav), config/settings.json, logs/system.log

STEP 13: Code Module 1 — Morse Code Decoder (morse.py)

- Import RPi.GPIO and define GPIO pins: DOT_PIN=17, DASH_PIN=27, CONFIRM_PIN=22.

- Set GPIO.setmode(GPIO.BCM) and GPIO.setup for each pin as INPUT.

- Create function detect_press() that measures how long a button is held (time.time).

- Short press (<400ms on DASH) = dot '.', Long press (>=400ms) = dash '-'.

- Create morse_to_char() dictionary mapping all Morse patterns to letters (A-Z, 0-9).

- Create decode_morse(sequence) function that looks up the pattern in the dictionary.

- Create function get_word() that collects letters until a word-gap timeout and returns the full word.

- Play beep.wav on each button press using pygame.mixer for audio confirmation.

STEP 14: Code Module 2 — OCR Reader (ocr.py)

- Import picamera2, cv2, pytesseract, PIL.Image.

- Function capture_image(): initialise picamera2, set resolution 1920x1080, capture to numpy array, save as temp.jpg.

- Function preprocess(image): convert to grayscale, apply cv2.adaptiveThreshold (ADAPTIVE_THRESH_GAUSSIAN_C), apply cv2.medianBlur to reduce noise.

- Function extract_text(lang='eng'): run pytesseract.image_to_string with lang parameter. Lang options: 'eng', 'hin', 'guj', 'eng+hin', 'eng+guj'.

- Function scan_and_read(language): calls capture → preprocess → extract → returns text string.

- Handle exceptions: if text is empty, play error tone and say 'No text detected, please try again'.

STEP 15: Code Module 3 — Text-to-Speech (tts.py)

- Import pyttsx3, gTTS, os, pygame.

- Function speak_offline(text, lang): use pyttsx3 engine. Set voice to eSpeak Hindi/Gujarati voice ID based on lang parameter.

- Function speak_online(text, lang): use gTTS(text=text, lang=lang_code, slow=False). Save to temp_audio.mp3. Play with pygame.mixer.

- Function speak(text, lang, online=True): try online first, fallback to offline on error (no internet).

- Language codes: English='en', Hindi='hi', Gujarati='gu'.

- Test function: speak('Hello, I am ready', 'en'). Should play from speaker.

STEP 16: Code Module 4 — Voice Input (voice.py)

- Import speech_recognition as sr.

- Initialize recognizer = sr.Recognizer().

- Function listen(lang='en-IN'): use sr.Microphone() as source. Say 'Speak now' via TTS. Call recognizer.listen(source, timeout=10).

- Call recognizer.recognize_google(audio, language=lang) to convert to text.

- Return transcribed text string. Catch sr.UnknownValueError and return None.

- Language codes for Google: English='en-IN', Hindi='hi-IN', Gujarati='gu-IN'.

STEP 17: Code Module 5 — AI Query / ChatGPT (ai_query.py)

- Import openai and load API key from config/settings.json.

- Set openai.api_key = config['openai_api_key'].

- Function ask_ai(prompt, context_lang='en'): call openai.chat.completions.create(model='gpt-3.5-turbo', messages=[{role:'user', content:prompt}]).

- Extract response: reply = response.choices[0].message.content.

- Return reply string.

- Wrap in try/except: on network error, return 'AI service unavailable. Please check internet connection.'

- NOTE: You need to get an OpenAI API key from platform.openai.com. Store key in settings.json, never hard-code in source.

STEP 18: Code Module 6 — Translator (translator.py)

- Import deep_translator or googletrans.Translator.

- Function translate(text, from_lang, to_lang): call translator.translate(text, src=from_lang, dest=to_lang).

- Return translated text string.

- Handle exception: if translation fails, return original text with audio message 'Translation failed'.

STEP 19: Code main.py — Bring All Modules Together

- Import all modules: morse, ocr, tts, voice, ai_query, translator.

- Load settings from config/settings.json.

- On startup: tts.speak('Welcome. Accessible Terminal Ready. Press Morse to select mode.', lang).

- Main loop: listen for Morse input from buttons.

- Morse 'A' (.-) = Mode A: SCAN. Call ocr.scan_and_read(lang) → tts.speak(result).

- Morse 'B' (-...) = Mode B: TYPE & AI. Collect word via morse.get_word() → ai_query.ask_ai(word) → tts.speak(answer).

- Morse 'C' (-.-.) = Mode C: VOICE. voice.listen() → ai_query.ask_ai(transcript) → tts.speak(answer).

- Morse 'D' (-..) = Mode D: TRANSLATE. Prompt source lang → target lang → translate text.

- Morse 'E' (.) = Mode E: GESTURE MODE. Launch gesture_control.py thread.

- Morse 'F' (..-.) = Mode F: OBJECT SCAN. Launch object_recognition.py.

- Morse 'G' (--.) = Mode G: GPS NAVIGATE. Launch gps_navigator.py.

- All modes announce themselves when selected: 'Scan mode active', 'Gesture mode active', etc.

## PHASE 4B: NEW FEATURE MODULES — CODING

STEP 20: Code Module 7 — Hand Gesture Recognition (gesture_control.py)

- pip3 install mediapipe

- Import cv2, mediapipe as mp, threading.

- Initialize: mp_hands = mp.solutions.hands.Hands(max_num_hands=1, min_detection_confidence=0.85).

- Open USB webcam: cap = cv2.VideoCapture(1)  [index 0 = Pi camera, index 1 = USB webcam].

- In a loop: read frame → convert BGR to RGB → process with mp_hands.process(frame_rgb).

- Extract landmark positions: hand_landmarks.landmark[mp.solutions.hands.HandLandmark.THUMB_TIP] etc.

- Create function classify_gesture(landmarks) → returns 'OPEN_PALM', 'THUMBS_UP', 'TWO_FINGERS', 'FIST', 'POINT_DOWN'.

- Use distance between landmark points to classify. Example: if all fingertip Y coords < wrist Y = OPEN_PALM.

- Function get_gesture(): run 10 frames, return most frequent gesture (majority vote) to avoid false triggers.

- Map gesture to mode string and return to main.py for mode switching.

- Run gesture_control as a daemon thread: threading.Thread(target=gesture_loop, daemon=True).start()

STEP 21: Code Module 8 — Emotion Detection (emotion_detector.py)

- pip3 install librosa scikit-learn soundfile speechbrain

- Import librosa, numpy, sklearn.svm, pickle.

- Function record_clip(duration=3): use PyAudio to record 3 seconds of audio to numpy array.

- Function extract_features(audio_array, sr=16000): call librosa.feature.mfcc(y=audio, sr=sr, n_mfcc=13). Also extract spectral_centroid and rms. Flatten and concatenate into single feature vector.

- Pre-train a simple SVM model on a small dataset of calm/stressed/confused voice samples (you can find free datasets on Kaggle: RAVDESS or CREMA-D).

- Save trained model: pickle.dump(clf, open('emotion_model.pkl', 'wb'))

- Function detect_emotion(audio): extract_features → load model → clf.predict(features) → return 'CALM', 'STRESSED', or 'CONFUSED'.

- In main.py: call detect_emotion() after every voice input. Track count. If CONFUSED x2: tts.set_rate(slow) and add 'Explain simply' to AI prompt.

STEP 22: Code Module 9 — Object Recognition (object_recognition.py)

- pip3 install ultralytics

- Download model: from ultralytics import YOLO; model = YOLO('yolov8n.pt')  [auto-downloads ~6MB on first run].

- For Google Coral acceleration: pip3 install pycoral tflite-runtime. Convert model to TFLite Edge TPU format using Coral's compiler.

- Function scan_objects(): capture frame from Pi camera → results = model(frame) → parse results.boxes.

- For each detected box: extract class name (results.names[int(box.cls)]) and confidence (box.conf).

- Calculate position: if box center_x < frame_width/3 → 'on your left'; if > 2/3 → 'on your right'; else → 'in front of you'.

- Build spoken string: 'I can see: [object] [position], [object] [position]...' Filter by confidence > 0.65.

- Function obstacle_alert(): run continuously in background thread. If any object box height > 40% of frame height → 'Warning: large object very close ahead.'

- Special cases: if class is 'person' → say 'A person is [position]'. If class is 'bottle' → 'A bottle is [position]'.

STEP 23: Code Module 10 — GPS Navigator (gps_navigator.py)

- Enable UART on Pi: sudo raspi-config → Interface Options → Serial → Disable login shell over serial → Enable serial hardware.

- pip3 install pyserial pynmea2 requests

- Connect NEO-6M: TX pin of GPS to GPIO 15 (RXD), RX pin of GPS to GPIO 14 (TXD), VCC to 3.3V, GND to GND.

- Function read_gps(): ser = serial.Serial('/dev/ttyAMA0', 9600, timeout=1). Read lines. Filter for '$GPRMC' or '$GPGGA'. Parse with pynmea2.parse(line). Return (latitude, longitude).

- Function get_address(lat, lon): call requests.get(f'https://nominatim.openstreetmap.org/reverse?lat={lat}&lon={lon}&format=json'). Extract display_name from JSON.

- Function navigate_to(destination, current_lat, current_lon): build prompt: f'I am at {address}. Give me simple walking directions to {destination} in numbered steps.' → send to AI → return steps list.

- In main loop: read current GPS. Speak 'You are at [address]'. If user gives destination via voice: navigate_to() → speak each step. Wait for user to say 'next' before reading next step.

- Offline fallback: if no internet, read GPS coordinates and say 'Navigation needs internet. Your coordinates are [lat], [lon].'

STEP 24: Code Module 11 — Offline AI / TinyLlama (offline_ai.py)

- pip3 install llama-cpp-python

- Download model: wget https://huggingface.co/TheBloke/TinyLlama-1.1B-Chat-v1.0-GGUF/resolve/main/tinyllama-1.1b-chat-v1.0.Q4_K_M.gguf -O /media/pi/USB/tinyllama.gguf

- This is a ~700MB download. Store on USB flash drive, not MicroSD.

- from llama_cpp import Llama; llm = Llama(model_path='/media/pi/USB/tinyllama.gguf', n_ctx=512, n_threads=4)

- Function ask_offline(prompt): response = llm(f'<human>: {prompt} <bot>:', max_tokens=200, stop=['<human>']). Return response['choices'][0]['text'].

- In ai_query.py: wrap the online ask_ai() in try/except. On failure: call offline_ai.ask_offline(prompt) as fallback.

- Speed note: TinyLlama on Pi 5 is faster than Pi 4 — expect 5-10 tokens/second. Set user expectation: 'Please wait, thinking...' via TTS.

- To improve speed: use n_threads=4 and keep context window small (n_ctx=256 for faster inference).

STEP 25: Code Module 12 — 🔒 Confidential Mode (privacy_mode.py)

- Import RPi.GPIO, subprocess, time.

- Define GPIO pin for PAM8403 amplifier enable/disable: AMP_ENABLE_PIN = 18. Set as GPIO.OUT.

- Run: aplay -l to list all audio devices. Note the card numbers for: (a) USB audio dongle → speaker, (b) bone conduction PAM8403 output.

- Store in config/settings.json: { 'speaker_alsa': 'hw:2,0', 'bone_alsa': 'hw:1,0', 'privacy_timeout': 8 }

- Function enable_bone_only(): GPIO.output(AMP_ENABLE_PIN, GPIO.HIGH). subprocess.run(['amixer', '-D', 'hw:2,0', 'set', 'Master', 'mute']). Sets default_tts_device = bone_alsa.

- Function enable_speaker_only(): GPIO.output(AMP_ENABLE_PIN, GPIO.LOW). subprocess.run(['amixer', '-D', 'hw:2,0', 'set', 'Master', 'unmute']). Sets default_tts_device = speaker_alsa.

- Class PrivateAudio (context manager): __enter__ calls enable_bone_only(), __exit__ calls enable_speaker_only(). Usage: with PrivateAudio(): tts.speak(text).

- Function ask_confidentiality(): call enable_bone_only() first. tts.speak('Is this confidential? Press 1 for Private, 2 for Speaker.', output=bone_alsa). Start timer (8 seconds). Wait for GPIO button press.

- If Button 1 (GPIO 17) pressed → return 'PRIVATE'. If Button 2 (GPIO 27) pressed → return 'NORMAL'. If timeout → return 'NORMAL'.

- In main.py: before EVERY tts.speak(scanned_text) or tts.speak(ai_response) call, ALWAYS run: mode = ask_confidentiality(). Then: if mode == 'PRIVATE': with PrivateAudio(): tts.speak(text). Else: tts.speak(text).

- Test with a piece of paper showing a bank account number: scan → press 1 → verify only bone conduction vibrates, speaker is silent.

STEP 26: Wire and Connect All Hardware Modules

- PAM8403 enable pin: Connect PAM8403 board's enable/shutdown pin to Pi 5 GPIO 18 (Pin 12) via a 1kΩ resistor.

- GPS Module wiring: GPS-TX → Pi 5 GPIO 15 (Pin 22, UART RX), GPS-RX → Pi 5 GPIO 14 (Pin 8, UART TX), GPS-VCC → 3.3V (Pin 17), GPS-GND → GND (Pin 20).

- Bone Conduction: USB audio card 3.5mm out → PAM8403 AIN. PAM8403 VCCIN → 5V (Pin 2), GND → GND. PAM8403 AOUT → bone conduction transducer wires.

- USB Webcam: plug into USB hub port. Verify: ls /dev/video* — should show /dev/video0 and /dev/video1.

- Google Coral: plug into USB 3.0 port (blue port on Pi 5). Verify: python3 -c 'from pycoral.utils import edgetpu; print(edgetpu.list_edge_tpus())'

- 128GB USB flash drive: plug into remaining USB 3.0 port. Format as ext4: sudo mkfs.ext4 /dev/sda1. Mount: sudo mount /dev/sda1 /media/pi/USB.

## PHASE 5: TESTING — ALL MODULES

STEP 26: Component-Level Testing (All Original + New)

- Test buttons alone: GPIO test script — press each button and print 'Button 1 pressed' to terminal.

- Test camera alone: libcamera-jpeg -o test.jpg — view the image file.

- Test OCR alone: place a printed page under camera, run ocr.scan_and_read('eng'). Check extracted text.

- Test TTS alone: run tts.speak('Testing speech output', 'en'). Should play from speaker.

- Test Morse decoder: tap buttons and check if correct letters appear on screen.

- Test USB webcam: python3 -c 'import cv2; cap=cv2.VideoCapture(1); ret,f=cap.read(); print(ret)' — should print True.

- Test MediaPipe gesture: run gesture_control.py, wave open palm at webcam. Terminal should print OPEN_PALM.

- Test GPS: connect module outdoors. python3 gps_test.py — should print latitude/longitude within 60 seconds.

- Test bone conduction: play test audio through PAM8403 amplifier — press transducer against wrist to feel vibration.

- Test Coral: python3 -c 'from pycoral.utils import edgetpu; print(edgetpu.list_edge_tpus())' — should list 1 device.

- Test YOLOv8: python3 -c 'from ultralytics import YOLO; m=YOLO("yolov8n.pt"); print("YOLO OK")'

- Test TinyLlama offline: ask a question with no internet connected. Response should come from local model.

- Fix each issue before moving to next.

STEP 27: Integration Testing

- Test OCR + TTS pipeline: scan document → text should be read aloud automatically.

- Test Morse → AI flow: type a question in Morse → AI response spoken.

- Test Voice → Emotion → AI flow: speak normally → AI responds. Speak confused → AI simplifies.

- Test Gesture → Mode switch: show OPEN_PALM → device enters Scan mode.

- Test Object Recognition: point camera at common objects → device names them aloud.

- Test GPS outdoors: ask 'Where am I?' → device speaks street address.

- Test Offline AI: disable WiFi → ask a question → TinyLlama responds.

- Test Confidential Mode — CRITICAL: scan a paper with a fake bank account number. When privacy prompt plays via bone conduction, press Button 1. Verify: speaker is completely silent, only bone transducer vibrates. Press transducer against your wrist — you should feel/hear the number. Check room — no one nearby should hear anything.

- Test Confidential Mode NORMAL path: same scan, press Button 2 → speaker plays, bone conduction silent.

- Test timeout: scan, wait 8 seconds without pressing → device defaults to speaker mode.

STEP 28: User Acceptance Testing (UAT)

- Invite a sighted person to blindfold themselves and use only buttons, gestures, voice, and audio.

- Ask them to: scan a book page, type their name in Morse, ask a question by voice, identify an object via camera.

- Ask them to: navigate to a location using GPS and voice, trigger emotion mode by sounding confused repeatedly.

- Record all confusion points. Adjust audio prompts, timing thresholds, and gesture sensitivity.

- Ideally test with an actual visually impaired person if accessible.

## PHASE 6: ASSEMBLY INTO ENCLOSURE

STEP 29: Prepare 3D Printed Shell

- 3D print top shell (orange) and base plate (navy) using the patent design dimensions.

- If 3D printer not available, contact a local 3D printing service — provide the STL files.

- Sand any rough edges inside the shell that could damage wiring.

- Verify all three button holes align exactly with your chosen tactile buttons.

- Verify camera aperture hole diameter matches Camera Module v2 lens size.

STEP 30: Mount Components Inside Shell

- Screw the 4 brass standoffs into the base plate using M2.5 screws.

- Place Raspberry Pi on standoffs and screw down with M2.5 screws — do not overtighten.

- Attach heatsinks to Pi CPU, RAM, and USB chip using thermal adhesive pads.

- Cable-tie all wiring neatly — button wires routed along the edge away from Pi vents.

- Attach the fan to the heatsink. Connect fan red wire to GPIO 5V pin, black wire to GND.

- Camera ribbon cable must be routed cleanly through the top shell's camera aperture.

- Place USB audio card inside the shell connected to Pi — route the audio jack to the side port.

STEP 31: Final Enclosure Close and Cable Management

- With everything mounted and tested (Pi powered on, all modules responding), shut down Pi cleanly: sudo shutdown now.

- Wait for the green LED to stop blinking before unplugging power.

- Align the top shell with base plate — ensure all vents and ports line up.

- Screw in the 8x M3 screws around the perimeter to close the shell.

- Power on again and run a final functional test before submitting the project.

## PHASE 7: AUTO-START ON BOOT

STEP 32: Make the App Start Automatically

- The blind user cannot type to start the program. It must launch on power-on.

- In terminal: sudo nano /etc/rc.local

- Before the 'exit 0' line, add: python3 /home/pi/blindterminal/main.py &

- Press Ctrl+X → Y → Enter to save.

- Reboot and verify: sudo reboot — program should start automatically after 60s boot.

- Audio message should play: 'Welcome. Accessible Terminal Ready.' without any login.

# SECTION 7: PRECAUTIONS & SAFETY

This section documents all safety measures required at each phase. Follow every precaution. Red alerts mark the highest-risk steps.

### 7.1  Electrical Safety

🚨 RED ALERT: GPIO Overvoltage Raspberry Pi GPIO pins operate at 3.3V logic. NEVER connect 5V directly to a GPIO pin — this will permanently destroy the chip. All button connections use 3.3V logic via pull-down resistors only.

🚨 RED ALERT: Short Circuit Risk Before closing the enclosure, check for any exposed bare wire ends that could touch the Pi board or each other. Use heat-shrink tubing or electrical tape on all exposed wire joins.

- Always power OFF the Pi completely before connecting or disconnecting any component.

- Use the official Raspberry Pi 5V 3A power supply only. Cheap chargers cause voltage drops, system crashes, and SD card corruption.

- The camera ribbon cable carries NO power but is fragile. Handle only from connector end.

- Never operate the device without the cooling fan connected — Pi 4 throttles and crashes at high temperature.

- GPS module uses 3.3V logic. Connect VCC to 3.3V GPIO pin only — NOT 5V. Connecting to 5V will permanently damage the NEO-6M module.

- PAM8403 amplifier for bone conduction: VCC must be 5V from GPIO, input signal from 3.5mm jack. Do NOT reverse wires — the IC will burn instantly.

- Confidential Mode GPIO pin (GPIO 18 for PAM8403 enable): ensure this pin is set LOW (amplifier OFF) at startup before any code runs. If the pin floats HIGH during boot, bone conduction may activate unexpectedly before the privacy prompt plays.

- Google Coral USB Accelerator runs hot during inference. Do not cover its metal heatsink surface inside the enclosure. Ensure airflow from the honeycomb vents reaches it.

- USB flash drive (for LLM model): never unplug while TinyLlama is loading or writing. Corruption will require re-downloading the 700MB model file.

### 7.2  Software & Data Safety

🚨 RED ALERT: API Key Security Your OpenAI API key is linked to a billing account. NEVER commit it to a public GitHub repo. Store it only in config/settings.json which must be in your .gitignore file.

- Always shut down Pi cleanly (sudo shutdown now) before unplugging power. Sudden power loss corrupts the MicroSD card, destroying the OS.

- Keep a backup of your code on GitHub and a second MicroSD with the OS image.

- The OpenAI API charges per usage. Set a spending limit on your OpenAI account dashboard.

### 7.3  Component Failure Risk Table

| Component | Failure Risk | Prevention |
| --- | --- | --- |
| MicroSD Card | HIGH — Corrupts on power loss | Always shutdown cleanly. Keep a backup image. |
| Camera Ribbon Cable | HIGH — Tears easily | Handle gently. Only touch plastic connector. Buy 1 spare. |
| Raspberry Pi (CPU) | MEDIUM — Overheating, GPIO damage | Install heatsinks + fan. Never connect 5V to GPIO. |

# SECTION 8: FINAL ACHIEVEMENT & OUTPUT

At the end of this project, the following deliverables will be completed and demonstrated:

### 8.1  Physical Deliverables

- One fully assembled Accessible Educational Terminal device inside the 3D-printed enclosure.

- All wiring professionally managed inside the shell — no exposed wires.

- Device is self-contained: power it on and it is ready to use in under 90 seconds.

### 8.2  Software Deliverables

- Complete Python codebase pushed to GitHub (github.com/parth-2005/morse-ai).

- README.md with full setup instructions for anyone to replicate the project.

- Configuration file for language, Morse timing, API settings.

### 8.3  Demonstrated Capabilities

| # | Demonstration Task | Expected Output |
| --- | --- | --- |
| 1 | Hold English text under camera and activate scan | Device reads text aloud in English clearly |
| 2 | Hold Gujarati text under camera and scan | Device reads Gujarati text in Gujarati language |
| 3 | Type 'HELLO' using Morse code buttons | Device announces each letter and spells the word |
| 4 | Use Morse to ask 'What is AI?' to ChatGPT | AI response is spoken aloud within 5 seconds |
| 5 | Speak a question aloud into microphone | Voice transcribed, AI responds, response spoken |
| 6 | Ask for translation of scanned text | English text translated and read in Hindi/Gujarati |
| 7 | Power off and on again | Device boots and is ready automatically — no keyboard needed |
| 8 | Show OPEN PALM gesture to webcam | Device enters Scan mode and announces 'Scan mode active' hands-free |
| 9 | Point camera at 3 random objects on a table | Device names all visible objects with their positions aloud within 2 seconds |
| 10 | Take device outdoors and ask 'Where am I?' | GPS coordinates resolved to street address, read aloud within 90 seconds of boot |
| 11 | Disconnect WiFi and ask AI a question | TinyLlama answers from local model — fully offline, no internet needed |
| 12 | Speak in confused/hesitant voice twice | Device detects stress, slows TTS speed, says 'Let me explain more simply' |
| 13 | Scan paper with bank account number — press Button 1 (PRIVATE) | Speaker completely silent. Only bone transducer vibrates. Account number heard only by user through skull. |
| 14 | Scan same paper — press Button 2 (NORMAL) | Content plays through speaker. Bone conduction silent. Full dual-channel separation demonstrated. |
| 15 | Scan OTP from phone screen — choose PRIVATE | OTP delivered privately through bone conduction. No bystander in the room can hear it. |

# SECTION 9: CONCLUSION

This project bridges a critical gap between visually impaired users and modern AI-powered technology. By combining a Raspberry Pi processing unit, camera-based OCR, tactile Morse code input, multilingual Text-to-Speech, ChatGPT integration, hand gesture recognition, emotion-adaptive AI, real-time object recognition, GPS navigation, and fully offline AI — all within a single 3D-printed enclosure at a student-buildable budget — the Accessible Educational Terminal is a genuinely novel assistive device.

The previous team (Parth and teammates) contributed the foundational hardware design patent, the enclosure concept, and initial Morse AI experiments. Dhruv Vaghela and Dax Patel's contribution is to complete the full software implementation, integrate six new intelligent features, conduct end-to-end testing, and deliver a working prototype.

The key innovations of this project are:

- Tactile Morse code input as a complete keyboard replacement — usable without sight, learnable within days.

- Multilingual OCR + TTS covering English, Hindi, and Gujarati — practical for students across India.

- Hand Gesture Recognition — controlling a computer without touching anything, using only natural hand movements.

- Emotion-Adaptive AI — a device that listens not just to what you say, but how you feel, and adjusts accordingly.

- Real-time Object Recognition — transforming the camera from a text reader into a full visual interpreter of the world.

- GPS Navigation — extending the device's usefulness from the classroom to the street.

- 100% Offline AI via TinyLlama — making the device deployable in rural India with no internet dependency and zero API costs.

- Bone Conduction Audio — a hardware innovation that keeps the user's ears free and aware of their surroundings.

- 🔒 Confidential Mode — dual audio routing that delivers sensitive data (bank OTPs, account numbers, passwords) exclusively through bone conduction, completely inaudible to bystanders. The privacy question itself is also delivered silently — true end-to-end private audio. This feature alone makes the device appropriate for banking, medical, and official use.

This project costs approximately Rs. 8,000–12,000 to build — far below any commercial assistive device — and could be manufactured at scale. The combination of features attempted here has not been achieved in a single student-built device in India. Beyond the lab, this represents a stepping stone toward a certified, deployable assistive technology product with real patent value.

ℹ️ Final Words for Dhruv Vaghela & Dax Patel You now have zero-to-hero clarity on every aspect of this project — original features and all six new intelligent additions. Start with Phase 1 (Flash OS) and work through each phase in order. Build and test one module at a time. Push code to GitHub regularly. The combination of features in this device is unprecedented at student level — own it.

— End of Master Plan Document —

Dhruv Vaghela & Dax Patel  |  CSR / Infineon Project  |  Guided by Rajiv Sir